# 14. STCKY Causal Analysis: S#!t That's Built at Heights

277. Safe access provided to elevated work area?  [List Radio]

Walking Surfaces | Stairs | Ramps:
Relentless Housekeeping - Access | Egress free from obstructions, tripping hazards
Landings of not less than 30 inches
Uniform riser height and tread depth
No hazardous projections: protruding nails
Proper Handrail Installed
For more information see:
https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.1052 (https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.1052)
Ladders –
Ladders - Relentless Housekeeping - Access | Egress free from obstructions, tripping hazards
In good condition
At least a Type 1A (rated for up to 300 pounds)
Constructed of nonconductive material
Setup at a 4:1 angle
Secured | tied off to prevent movement
Extend 3ft beyond landing area or have “pass through” gate
Protected from fall exposure by means of a controlled access point
Be used per the manufacturer’s instructions
For more information see:
https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.1053 (https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.1053)
Elevated work platform
Inspected and maintained
Designed for intended use
Fall protection system in place

○ Yes

○ No

○ Not applicable

278. Safe access - Corrective Actions  [List Checkbox]

| ☐ Walking Surfaces - Relentless Housekeeping - Access ¦ Egress free from obstructions, tripping hazards | ☐ Install Proper Handrail |
| --- | --- |
| ☐ Ladders - Relentless Housekeeping - Access ¦ Egress free from obstructions, tripping hazards | ☐ Replace ladder with at least a Type 1A (rated for up to 300 pounds) |
| ☐ Replace with ladder Constructed of nonconductive material | ☐ Install ladder at a 4:1 angle |
| ☐ Secure ¦ tie off ladder to prevent movement | ☐ Extend ladder 3ft beyond landing area or install “pass through” gate |
| ☐ Protect workers from fall exposure by means of a controlled access point | ☐ MEWP - Review and train per the manufacturer’s instructions |
| ☐ MEWP - Inspect and maintain elevated work platform | ☐ MEWP - Use per intended design |
| ☐ MEWP - Install fall protection system |  |

279. Reason for lack of Safe Access  [List Radio]

○ Safe access hazard(s) not recognized - Hazard(s) Not Recognized Hazard(s) not recognized by employees involved in the task. Example: Dropped object hazard(s) not recognized by the crew.

○ Safe access control(s) not identified - Control(s) Not Identified Control(s) not identified for the recognized hazard(s) Example: The crew did not identify the need for tethers, C.A.Z. (Controlled Access Zone), etc., to protect from recognized dropped object hazards.

○ Safe access control(s) not implemented - Control(s) Not Implemented Identified control(s) were not implemented. Example: C.A.Z. identified as a control for dropped object hazard(s), but not established (implemented).

○ Safe access control(s) not executed - Control(s) Not Executed Implemented control(s) were not properly executed. Example: C.A.Z. was established but not enforced (executed).

280. What prevented the hazard from being recognized? Select all that apply.  [List Checkbox]

| ☐ Primary STCKY Energy not recognized | ☐ Lack of Knowledge |
| --- | --- |
| ☐ Inadequate training | ☐ Lack of awareness |
| ☐ Insufficient risk assessment | ☐ Rushed planning or inadequate preparation |
| ☐ Lack of communication | ☐ Inadequate hazard identification process |
| ☐ Unclear responsibilities | ☐ Assumptions and biases |
| ☐ External factors |  |

281. What prevented the control from being identified? Select all that apply.  [List Checkbox]

| ☐ Lack of familiarity with specific high energy hazards - The project team is not familiar with the specific high energy hazards present in the task, i.e. new or unique situation or work process. | ☐ Inadequate training on high energy controls - Team members have not received sufficient training on identifying, implementing, and executing high energy controls. |
| --- | --- |
| ☐ Insufficient risk assessment - The team has not conducted a thorough risk assessment to identify all potential high energy hazards and determine appropriate controls. | ☐ Lack of available control options - The team struggles to find suitable control measures for mitigating the identified high energy hazards, leading to delays or inadequate solutions. |
| ☐ Resource constraints - Limited budget or access to equipment prevented the implementation of high energy controls | ☐ Time constraints - Project deadlines and tight schedules pressure the team to overlook or rush the identification of high energy controls. |
| ☐ Risk Tolerance - The team has dealt with similar high energy hazards in the past without incident and assumed the risks are manageable without additional controls. | ☐ Emphasis on production - A focus on productivity and meeting targets led to neglecting the thorough identification of high energy controls. |
| ☐ Lack of communication and collaboration - Team members did not communicate effectively, critical information about high energy hazards and controls were missed. | ☐ Failure to learn from past incidents - The team did not learn from previous incidents involving similar high energy hazards, leading to repeated oversights in control identification. |
| ☐ Complexity of the construction task - Highly complex task with multiple high energy hazards, made it challenging for the team to identify and implement appropriate controls for each hazard. |  |

282. What prevented the control from being implemented? Select all that apply.  [List Checkbox]

| ☐ Cost considerations - High energy controls are expensive to acquire and install, and the project team prioritized cost savings over worker safety. | ☐ Lack of understanding or awareness - The team did not fully comprehend the severity of the hazard or the potential consequences of not implementing the high energy controls. |
| --- | --- |
| ☐ Resistance to change - Introduction of controls or procedures was met with resistance from workers or management who prefer familiar practices. | ☐ Inadequate planning - The team identified the need for high energy control but failed to develop a proper plan for their implementation. |
| ☐ Resource constraints - The team lacked the necessary resources, such as equipment or skilled personnel, to implement the high energy controls effectively. | ☐ Time constraints - Project timelines or deadlines did not allow for implementing high energy controls properly. |
| ☐ Lack of leadership support - The absence of leadership support in prioritizing control implementation made it challenging for the team to effectively implement the controls. | ☐ Perceived inconvenience - Workers considered the high energy controls as inconvenient or cumbersome, leading to resistance in their implementation. |
| ☐ Risk Tolerance - The project team believes that the hazard is manageable without additional controls or that accidents are unlikely to happen. | ☐ Inadequate training - Workers lack the necessary training on how to use or maintain the high energy controls effectively. |
| ☐ Miscommunication or misinterpretation - Instructions or guidelines for implementing the controls have not been communicated clearly or were misunderstood by the team |  |

283. What prevented the control from being executed? Select all that apply.  [List Checkbox]

| ☐ Inadequate training - The team have not received sufficient training on how to use the high energy control properly | ☐ Lack of supervision - Lacking supervision, workers did not follow the correct procedures for using the high energy control. |
| --- | --- |
| ☐ Risk Tolerance - Workers became complacent over time and deviated from established procedures | ☐ Lack of accountability - No consequences for non-compliance with high energy control procedures |
| ☐ Failure to maintain equipment - Lack of regular maintenance and/or inspections of the high energy control led to malfunction and/or reduced effectiveness. | ☐ Miscommunication or misunderstandings - Poor communication between team members resulted in misunderstandings about how to execute the high energy control. |
| ☐ Production pressures - Workers prioritized production over control execution, due to tight project schedule or deadlines, leading to incomplete execution of the high energy control. | ☐ Resource constraints - The team lacked the necessary resources to ensure high energy control execution. |
| ☐ Lack of awareness - Workers did understand the importance of high energy control or the potential consequences of not executing it effectively. | ☐ Worker fatigue - Fatigued workers were not alert or attentive when executing high energy controls. |
| ☐ Competing priorities - Other tasks or issues diverted the team’s attention away from executing the high energy control. | ☐ Willful disregard - Intentional neglect or avoidance of necessary actions or procedures |

284. Fall prevention measures adequate?  [List Radio]

○ Yes

○ No

○ Not applicable

285. Fall Prevention - Corrective Actions  [List Checkbox]

| ☐ Install Top rails at 42” | ☐ Install Mid-rails at 21” |
| --- | --- |
| ☐ Tension Wire rope < 3” deflection | ☐ Install guard rail system capable of withstanding 200lbs. of force |
| ☐ Install rated hole covering or standard guardrail system to protect openings larger than 2" | ☐ Ensure hole coverings are secured to prevent displacement |
| ☐ Replace hole covering with one capable of supporting 2X max. intended load | ☐ Mark, using High Visible paint, “HOLE” or “COVER” |
| ☐ Cal/OSHA - requires hole covers to bear a pressure sensitized, painted, or stenciled sign with legible letters not less than one inch high, stating: “Opening--Do Not Remove.” | ☐ Install guardrail system for holes ¦openings > greater than 24” wide. |
| ☐ Ensure vertical supports max. 8’ apart with top rail, midrail, and toe boards. | ☐ Verify Employees trained on fall protection procedures |
| ☐ Installed C.A.Z. min. 6’ back from edge of deck ¦ floor ¦ wall opening. | ☐ Ensure 100% Fall protection required when employees cross flagging |

286. Reason for lack of Fall Prevention  [List Radio]

○ Fall Prevention hazard(s) not recognized - Hazard(s) Not Recognized Hazard(s) not recognized by employees involved in the task. Example: Dropped object hazard(s) not recognized by the crew.

○ Fall Prevention control(s) not identified - Control(s) Not Identified Control(s) not identified for the recognized hazard(s) Example: The crew did not identify the need for tethers, C.A.Z. (Controlled Access Zone), etc., to protect from recognized dropped object hazards.

○ Fall Prevention control(s) not implemented - Control(s) Not Implemented Identified control(s) were not implemented. Example: C.A.Z. identified as a control for dropped object hazard(s), but not established (implemented).

○ Fall Prevention control(s) not executed - Control(s) Not Executed Implemented control(s) were not properly executed. Example: C.A.Z. was established but not enforced (executed).

287. What prevented the hazard from being recognized? Select all that apply.  [List Checkbox]

| ☐ Primary STCKY Energy not recognized | ☐ Lack of Knowledge |
| --- | --- |
| ☐ Inadequate training | ☐ Lack of awareness |
| ☐ Insufficient risk assessment | ☐ Rushed planning or inadequate preparation |
| ☐ Lack of communication | ☐ Inadequate hazard identification process |
| ☐ Unclear responsibilities | ☐ Assumptions and biases |
| ☐ External factors |  |

288. What prevented the control from being identified? Select all that apply.  [List Checkbox]

| ☐ Lack of familiarity with specific high energy hazards - The project team is not familiar with the specific high energy hazards present in the task, i.e. new or unique situation or work process. | ☐ Inadequate training on high energy controls - Team members have not received sufficient training on identifying, implementing, and executing high energy controls. |
| --- | --- |
| ☐ Insufficient risk assessment - The team has not conducted a thorough risk assessment to identify all potential high energy hazards and determine appropriate controls. | ☐ Lack of available control options - The team struggles to find suitable control measures for mitigating the identified high energy hazards, leading to delays or inadequate solutions. |
| ☐ Resource constraints - Limited budget or access to equipment prevented the implementation of high energy controls | ☐ Time constraints - Project deadlines and tight schedules pressure the team to overlook or rush the identification of high energy controls. |
| ☐ Risk Tolerance - The team has dealt with similar high energy hazards in the past without incident and assumed the risks are manageable without additional controls. | ☐ Emphasis on production - A focus on productivity and meeting targets led to neglecting the thorough identification of high energy controls. |
| ☐ Lack of communication and collaboration - Team members did not communicate effectively, critical information about high energy hazards and controls were missed. | ☐ Failure to learn from past incidents - The team did not learn from previous incidents involving similar high energy hazards, leading to repeated oversights in control identification. |
| ☐ Complexity of the construction task - Highly complex task with multiple high energy hazards, made it challenging for the team to identify and implement appropriate controls for each hazard. |  |

289. What prevented the control from being implemented? Select all that apply.  [List Checkbox]

| ☐ Cost considerations - High energy controls are expensive to acquire and install, and the project team prioritized cost savings over worker safety. | ☐ Lack of understanding or awareness - The team did not fully comprehend the severity of the hazard or the potential consequences of not implementing the high energy controls. |
| --- | --- |
| ☐ Resistance to change - Introduction of controls or procedures was met with resistance from workers or management who prefer familiar practices. | ☐ Inadequate planning - The team identified the need for high energy control but failed to develop a proper plan for their implementation. |
| ☐ Resource constraints - The team lacked the necessary resources, such as equipment or skilled personnel, to implement the high energy controls effectively. | ☐ Time constraints - Project timelines or deadlines did not allow for implementing high energy controls properly. |
| ☐ Lack of leadership support - The absence of leadership support in prioritizing control implementation made it challenging for the team to effectively implement the controls. | ☐ Perceived inconvenience - Workers considered the high energy controls as inconvenient or cumbersome, leading to resistance in their implementation. |
| ☐ Risk Tolerance - The project team believes that the hazard is manageable without additional controls or that accidents are unlikely to happen. | ☐ Inadequate training - Workers lack the necessary training on how to use or maintain the high energy controls effectively. |
| ☐ Miscommunication or misinterpretation - Instructions or guidelines for implementing the controls have not been communicated clearly or were misunderstood by the team |  |

290. What prevented the control from being executed? Select all that apply.  [List Checkbox]

| ☐ Inadequate training - The team have not received sufficient training on how to use the high energy control properly | ☐ Lack of supervision - Lacking supervision, workers did not follow the correct procedures for using the high energy control. |
| --- | --- |
| ☐ Risk Tolerance - Workers became complacent over time and deviated from established procedures | ☐ Lack of accountability - No consequences for non-compliance with high energy control procedures |
| ☐ Failure to maintain equipment - Lack of regular maintenance and/or inspections of the high energy control led to malfunction and/or reduced effectiveness. | ☐ Miscommunication or misunderstandings - Poor communication between team members resulted in misunderstandings about how to execute the high energy control. |
| ☐ Production pressures - Workers prioritized production over control execution, due to tight project schedule or deadlines, leading to incomplete execution of the high energy control. | ☐ Resource constraints - The team lacked the necessary resources to ensure high energy control execution. |
| ☐ Lack of awareness - Workers did understand the importance of high energy control or the potential consequences of not executing it effectively. | ☐ Worker fatigue - Fatigued workers were not alert or attentive when executing high energy controls. |
| ☐ Competing priorities - Other tasks or issues diverted the team’s attention away from executing the high energy control. | ☐ Willful disregard - Intentional neglect or avoidance of necessary actions or procedures |

291. Personal fall protection properly used?  [List Radio]

○ Yes

○ No

○ Not applicable

292. Personal Fall Protection - Corrective Actions  [List Checkbox]

☐ Select/Install anchor point rated for 5000 lbs or greater

☐ Select leading edge rated lanyard

☐ Ensure fall harness properly donned and inspected

☐ Ensure 100% Tie off

☐ Ensure adequate fall clearance from lower level ¦ obstructions

☐ Review rescue plan / ensure rescue equipment in place

293. Reason for lack of Personal Fall Protection  [List Radio]

○ Personal Fall Protection hazard(s) not recognized - Hazard(s) Not Recognized Hazard(s) not recognized by employees involved in the task. Example: Dropped object hazard(s) not recognized by the crew.

○ Personal Fall Protection control(s) not identified - Control(s) Not Identified Control(s) not identified for the recognized hazard(s) Example: The crew did not identify the need for tethers, C.A.Z. (Controlled Access Zone), etc., to protect from recognized dropped object hazards.

○ Personal Fall Protection control(s) not implemented - Control(s) Not Implemented Identified control(s) were not implemented. Example: C.A.Z. identified as a control for dropped object hazard(s), but not established (implemented).

○ Personal Fall Protection control(s) not executed - Control(s) Not Executed Implemented control(s) were not properly executed. Example: C.A.Z. was established but not enforced (executed).

294. What prevented the hazard from being recognized? Select all that apply.  [List Checkbox]

| ☐ Primary STCKY Energy not recognized | ☐ Lack of Knowledge |
| --- | --- |
| ☐ Inadequate training | ☐ Lack of awareness |
| ☐ Insufficient risk assessment | ☐ Rushed planning or inadequate preparation |
| ☐ Lack of communication | ☐ Inadequate hazard identification process |
| ☐ Unclear responsibilities | ☐ Assumptions and biases |
| ☐ External factors |  |

295. What prevented the control from being identified? Select all that apply.  [List Checkbox]

| ☐ Lack of familiarity with specific high energy hazards - The project team is not familiar with the specific high energy hazards present in the task, i.e. new or unique situation or work process. | ☐ Inadequate training on high energy controls - Team members have not received sufficient training on identifying, implementing, and executing high energy controls. |
| --- | --- |
| ☐ Insufficient risk assessment - The team has not conducted a thorough risk assessment to identify all potential high energy hazards and determine appropriate controls. | ☐ Lack of available control options - The team struggles to find suitable control measures for mitigating the identified high energy hazards, leading to delays or inadequate solutions. |
| ☐ Resource constraints - Limited budget or access to equipment prevented the implementation of high energy controls | ☐ Time constraints - Project deadlines and tight schedules pressure the team to overlook or rush the identification of high energy controls. |
| ☐ Risk Tolerance - The team has dealt with similar high energy hazards in the past without incident and assumed the risks are manageable without additional controls. | ☐ Emphasis on production - A focus on productivity and meeting targets led to neglecting the thorough identification of high energy controls. |
| ☐ Lack of communication and collaboration - Team members did not communicate effectively, critical information about high energy hazards and controls were missed. | ☐ Failure to learn from past incidents - The team did not learn from previous incidents involving similar high energy hazards, leading to repeated oversights in control identification. |
| ☐ Complexity of the construction task - Highly complex task with multiple high energy hazards, made it challenging for the team to identify and implement appropriate controls for each hazard. |  |

296. What prevented the control from being implemented? Select all that apply.  [List Checkbox]

| ☐ Cost considerations - High energy controls are expensive to acquire and install, and the project team prioritized cost savings over worker safety. | ☐ Lack of understanding or awareness - The team did not fully comprehend the severity of the hazard or the potential consequences of not implementing the high energy controls. |
| --- | --- |
| ☐ Resistance to change - Introduction of controls or procedures was met with resistance from workers or management who prefer familiar practices. | ☐ Inadequate planning - The team identified the need for high energy control but failed to develop a proper plan for their implementation. |
| ☐ Resource constraints - The team lacked the necessary resources, such as equipment or skilled personnel, to implement the high energy controls effectively. | ☐ Time constraints - Project timelines or deadlines did not allow for implementing high energy controls properly. |
| ☐ Lack of leadership support - The absence of leadership support in prioritizing control implementation made it challenging for the team to effectively implement the controls. | ☐ Perceived inconvenience - Workers considered the high energy controls as inconvenient or cumbersome, leading to resistance in their implementation. |
| ☐ Risk Tolerance - The project team believes that the hazard is manageable without additional controls or that accidents are unlikely to happen. | ☐ Inadequate training - Workers lack the necessary training on how to use or maintain the high energy controls effectively. |
| ☐ Miscommunication or misinterpretation - Instructions or guidelines for implementing the controls have not been communicated clearly or were misunderstood by the team |  |

297. What prevented the control from being executed? Select all that apply.  [List Checkbox]

| ☐ Inadequate training - The team have not received sufficient training on how to use the high energy control properly | ☐ Lack of supervision - Lacking supervision, workers did not follow the correct procedures for using the high energy control. |
| --- | --- |
| ☐ Risk Tolerance - Workers became complacent over time and deviated from established procedures | ☐ Lack of accountability - No consequences for non-compliance with high energy control procedures |
| ☐ Failure to maintain equipment - Lack of regular maintenance and/or inspections of the high energy control led to malfunction and/or reduced effectiveness. | ☐ Miscommunication or misunderstandings - Poor communication between team members resulted in misunderstandings about how to execute the high energy control. |
| ☐ Production pressures - Workers prioritized production over control execution, due to tight project schedule or deadlines, leading to incomplete execution of the high energy control. | ☐ Resource constraints - The team lacked the necessary resources to ensure high energy control execution. |
| ☐ Lack of awareness - Workers did understand the importance of high energy control or the potential consequences of not executing it effectively. | ☐ Worker fatigue - Fatigued workers were not alert or attentive when executing high energy controls. |
| ☐ Competing priorities - Other tasks or issues diverted the team’s attention away from executing the high energy control. | ☐ Willful disregard - Intentional neglect or avoidance of necessary actions or procedures |

298. Dropped object protection in place?  [List Radio]

○ Yes

○ No

○ Not applicable

299. Dropped object protection - Corrective Actions  [List Checkbox]

| ☐ Tether tools and materials while working near leading edge | ☐ Remove unneeded tools from elevated work area |
| --- | --- |
| ☐ Establish C.A.Z. with physical barriers with adequate signage | ☐ Remove workers from hazard area, set up adequate C.A.Z. |
| ☐ Designate spotter for C.A.Z. when signage is not being used | ☐ Secure/Net material hoisted over active work areas |
| ☐ Repair ¦ Install netting ¦ screening | ☐ Install overhead protection at entrances/exits |
| ☐ Remove ¦ secure ¦ distance materials within 6ft. of leading edge |  |

300. Reason for lack of Dropped Object Protection  [List Radio]

○ Dropped Object hazard(s) not recognized - Hazard(s) Not Recognized Hazard(s) not recognized by employees involved in the task. Example: Dropped object hazard(s) not recognized by the crew.

○ Dropped Object control(s) not identified - Control(s) Not Identified Control(s) not identified for the recognized hazard(s) Example: The crew did not identify the need for tethers, C.A.Z. (Controlled Access Zone), etc., to protect from recognized dropped object hazards.

○ Dropped Object control(s) not implemented - Control(s) Not Implemented Identified control(s) were not implemented. Example: C.A.Z. identified as a control for dropped object hazard(s), but not established (implemented).

○ Dropped Object control(s) not executed - Control(s) Not Executed Implemented control(s) were not properly executed. Example: C.A.Z. was established but not enforced (executed).

301. What prevented the hazard from being recognized? Select all that apply.  [List Checkbox]

| ☐ Primary STCKY Energy not recognized | ☐ Lack of Knowledge |
| --- | --- |
| ☐ Inadequate training | ☐ Lack of awareness |
| ☐ Insufficient risk assessment | ☐ Rushed planning or inadequate preparation |
| ☐ Lack of communication | ☐ Inadequate hazard identification process |
| ☐ Unclear responsibilities | ☐ Assumptions and biases |
| ☐ External factors |  |

302. What prevented the control from being identified? Select all that apply.  [List Checkbox]

| ☐ Lack of familiarity with specific high energy hazards - The project team is not familiar with the specific high energy hazards present in the task, i.e. new or unique situation or work process. | ☐ Inadequate training on high energy controls - Team members have not received sufficient training on identifying, implementing, and executing high energy controls. |
| --- | --- |
| ☐ Insufficient risk assessment - The team has not conducted a thorough risk assessment to identify all potential high energy hazards and determine appropriate controls. | ☐ Lack of available control options - The team struggles to find suitable control measures for mitigating the identified high energy hazards, leading to delays or inadequate solutions. |
| ☐ Resource constraints - Limited budget or access to equipment prevented the implementation of high energy controls | ☐ Time constraints - Project deadlines and tight schedules pressure the team to overlook or rush the identification of high energy controls. |
| ☐ Risk Tolerance - The team has dealt with similar high energy hazards in the past without incident and assumed the risks are manageable without additional controls. | ☐ Emphasis on production - A focus on productivity and meeting targets led to neglecting the thorough identification of high energy controls. |
| ☐ Lack of communication and collaboration - Team members did not communicate effectively, critical information about high energy hazards and controls were missed. | ☐ Failure to learn from past incidents - The team did not learn from previous incidents involving similar high energy hazards, leading to repeated oversights in control identification. |
| ☐ Complexity of the construction task - Highly complex task with multiple high energy hazards, made it challenging for the team to identify and implement appropriate controls for each hazard. |  |

303. What prevented the control from being implemented? Select all that apply.  [List Checkbox]

| ☐ Cost considerations - High energy controls are expensive to acquire and install, and the project team prioritized cost savings over worker safety. | ☐ Lack of understanding or awareness - The team did not fully comprehend the severity of the hazard or the potential consequences of not implementing the high energy controls. |
| --- | --- |
| ☐ Resistance to change - Introduction of controls or procedures was met with resistance from workers or management who prefer familiar practices. | ☐ Inadequate planning - The team identified the need for high energy control but failed to develop a proper plan for their implementation. |
| ☐ Resource constraints - The team lacked the necessary resources, such as equipment or skilled personnel, to implement the high energy controls effectively. | ☐ Time constraints - Project timelines or deadlines did not allow for implementing high energy controls properly. |
| ☐ Lack of leadership support - The absence of leadership support in prioritizing control implementation made it challenging for the team to effectively implement the controls. | ☐ Perceived inconvenience - Workers considered the high energy controls as inconvenient or cumbersome, leading to resistance in their implementation. |
| ☐ Risk Tolerance - The project team believes that the hazard is manageable without additional controls or that accidents are unlikely to happen. | ☐ Inadequate training - Workers lack the necessary training on how to use or maintain the high energy controls effectively. |
| ☐ Miscommunication or misinterpretation - Instructions or guidelines for implementing the controls have not been communicated clearly or were misunderstood by the team |  |

304. What prevented the control from being executed? Select all that apply.  [List Checkbox]

| ☐ Inadequate training - The team have not received sufficient training on how to use the high energy control properly | ☐ Lack of supervision - Lacking supervision, workers did not follow the correct procedures for using the high energy control. |
| --- | --- |
| ☐ Risk Tolerance - Workers became complacent over time and deviated from established procedures | ☐ Lack of accountability - No consequences for non-compliance with high energy control procedures |
| ☐ Failure to maintain equipment - Lack of regular maintenance and/or inspections of the high energy control led to malfunction and/or reduced effectiveness. | ☐ Miscommunication or misunderstandings - Poor communication between team members resulted in misunderstandings about how to execute the high energy control. |
| ☐ Production pressures - Workers prioritized production over control execution, due to tight project schedule or deadlines, leading to incomplete execution of the high energy control. | ☐ Resource constraints - The team lacked the necessary resources to ensure high energy control execution. |
| ☐ Lack of awareness - Workers did understand the importance of high energy control or the potential consequences of not executing it effectively. | ☐ Worker fatigue - Fatigued workers were not alert or attentive when executing high energy controls. |
| ☐ Competing priorities - Other tasks or issues diverted the team’s attention away from executing the high energy control. | ☐ Willful disregard - Intentional neglect or avoidance of necessary actions or procedures |