# 21. STCKY Causal Analysis: S#!t That Lifts

453. Load Handling Equipment Classification  [List Radio]

○ Cranes

○ Other Load Handling Devices (equipment with forks, hoists, etc)

454. Required Crane set up procedures in place?  [List Radio]

Underground hazards identified and marked
Ground rated for load
Ground conditions firm, drained, and graded
Outriggers extended as required by load chart
Floats level, cribbing/matting sufficient
Swing radius protection in place

○ Yes

○ No

455. Cranes Set Up - Corrective Actions  [List Checkbox]

☐ Identify and mark underground hazards

☐ Verify ground is rated for load

☐ Ensure ground conditions are safe for operation

☐ Extend outriggers as required by load chart

☐ Ensure Floats are level and cribbing/matting is sufficient

☐ Protect swing radius barricade

456. Reason for lack of crane set up  [List Radio]

○ Crane Set Up hazard(s) not recognized

○ Crane Set Up control(s) not identified

○ Crane Set Up control(s) not implemented

○ Crane Set Up control(s) not executed

457. What prevented the hazard from being recognized? Select all that apply.  [List Checkbox]

| ☐ Primary STCKY Energy not recognized | ☐ Lack of Knowledge |
| --- | --- |
| ☐ Inadequate training | ☐ Lack of awareness |
| ☐ Insufficient risk assessment | ☐ Rushed planning or inadequate preparation |
| ☐ Lack of communication | ☐ Inadequate hazard identification process |
| ☐ Unclear responsibilities | ☐ Assumptions and biases |
| ☐ External factors |  |

458. What prevented the control from being identified? Select all that apply.  [List Checkbox]

| ☐ Lack of familiarity with specific high energy hazards - The project team is not familiar with the specific high energy hazards present in the task, i.e. new or unique situation or work process. | ☐ Inadequate training on high energy controls - Team members have not received sufficient training on identifying, implementing, and executing high energy controls. |
| --- | --- |
| ☐ Insufficient risk assessment - The team has not conducted a thorough risk assessment to identify all potential high energy hazards and determine appropriate controls. | ☐ Lack of available control options - The team struggles to find suitable control measures for mitigating the identified high energy hazards, leading to delays or inadequate solutions. |
| ☐ Resource constraints - Limited budget or access to equipment prevented the implementation of high energy controls | ☐ Time constraints - Project deadlines and tight schedules pressure the team to overlook or rush the identification of high energy controls. |
| ☐ Risk Tolerance - The team has dealt with similar high energy hazards in the past without incident and assumed the risks are manageable without additional controls. | ☐ Emphasis on production - A focus on productivity and meeting targets led to neglecting the thorough identification of high energy controls. |
| ☐ Lack of communication and collaboration - Team members did not communicate effectively, critical information about high energy hazards and controls were missed. | ☐ Failure to learn from past incidents - The team did not learn from previous incidents involving similar high energy hazards, leading to repeated oversights in control identification. |
| ☐ Complexity of the construction task - Highly complex task with multiple high energy hazards, made it challenging for the team to identify and implement appropriate controls for each hazard. |  |

459. What prevented the control from being implemented? Select all that apply.  [List Checkbox]

| ☐ Cost considerations - High energy controls are expensive to acquire and install, and the project team prioritized cost savings over worker safety. | ☐ Lack of understanding or awareness - The team did not fully comprehend the severity of the hazard or the potential consequences of not implementing the high energy controls. |
| --- | --- |
| ☐ Resistance to change - Introduction of controls or procedures was met with resistance from workers or management who prefer familiar practices. | ☐ Inadequate planning - The team identified the need for high energy control but failed to develop a proper plan for their implementation. |
| ☐ Resource constraints - The team lacked the necessary resources, such as equipment or skilled personnel, to implement the high energy controls effectively. | ☐ Time constraints - Project timelines or deadlines did not allow for implementing high energy controls properly. |
| ☐ Lack of leadership support - The absence of leadership support in prioritizing control implementation made it challenging for the team to effectively implement the controls. | ☐ Perceived inconvenience - Workers considered the high energy controls as inconvenient or cumbersome, leading to resistance in their implementation. |
| ☐ Risk Tolerance - The project team believes that the hazard is manageable without additional controls or that accidents are unlikely to happen. | ☐ Inadequate training - Workers lack the necessary training on how to use or maintain the high energy controls effectively. |
| ☐ Miscommunication or misinterpretation - Instructions or guidelines for implementing the controls have not been communicated clearly or were misunderstood by the team |  |

460. What prevented the control from being executed? Select all that apply.  [List Checkbox]

| ☐ Inadequate training - The team have not received sufficient training on how to use the high energy control properly | ☐ Lack of supervision - Lacking supervision, workers did not follow the correct procedures for using the high energy control. |
| --- | --- |
| ☐ Risk Tolerance - Workers became complacent over time and deviated from established procedures | ☐ Lack of accountability - No consequences for non-compliance with high energy control procedures |
| ☐ Failure to maintain equipment - Lack of regular maintenance and/or inspections of the high energy control led to malfunction and/or reduced effectiveness. | ☐ Miscommunication or misunderstandings - Poor communication between team members resulted in misunderstandings about how to execute the high energy control. |
| ☐ Production pressures - Workers prioritized production over control execution, due to tight project schedule or deadlines, leading to incomplete execution of the high energy control. | ☐ Resource constraints - The team lacked the necessary resources to ensure high energy control execution. |
| ☐ Lack of awareness - Workers did understand the importance of high energy control or the potential consequences of not executing it effectively. | ☐ Worker fatigue - Fatigued workers were not alert or attentive when executing high energy controls. |
| ☐ Competing priorities - Other tasks or issues diverted the team’s attention away from executing the high energy control. | ☐ Willful disregard - Intentional neglect or avoidance of necessary actions or procedures |

461. Safe procedures for work near overhead utilities? (Cranes)  [List Radio]

Overhead Utilities:
For work near powerlines:
All of the following must be met:
Planning meeting (discuss de-energizing lines)
Signs and barricades
Non-conductive tag lines
Warning lines erected
Spotter required
Encroachment Precautions for operations up to 350kV:
At Least One of These Additional Measures
Proximity alarm
Range control warning device
Range control prevention device
Insulating link
Table A – Minimum Powerline Clearance Distances – Operation
Voltage
Distance
0 to 50 kV
10 ft
50 to 200 kV
15 ft
200 to 350 kV
20 ft
350 to 500 kV
25 ft
500 to 750 kV
35 ft
750 to 1000 kV
45 ft

○ Yes

○ No

○ Not applicable

462. Work near overhead utilities - Corrective Actions  [List Checkbox]

☐ Conduct planning meeting (discuss de-energizing lines)

☐ Erect signs and barricades

☐ Erect warning lines

☐ Provide Non-conductive tag lines

☐ Designate spotter as required

☐ Ensure minimum clearance distance per Table A

463. Reason for lack of protection near overhead utilities (Cranes)  [List Radio]

○ Overhead Utilities hazard(s) not recognized

○ Overhead Utilities control(s) not identified

○ Overhead Utilities control(s) not implemented

○ Overhead Utilities control(s) not executed

464. What prevented the hazard from being recognized? Select all that apply.  [List Checkbox]

| ☐ Primary STCKY Energy not recognized | ☐ Lack of Knowledge |
| --- | --- |
| ☐ Inadequate training | ☐ Lack of awareness |
| ☐ Insufficient risk assessment | ☐ Rushed planning or inadequate preparation |
| ☐ Lack of communication | ☐ Inadequate hazard identification process |
| ☐ Unclear responsibilities | ☐ Assumptions and biases |
| ☐ External factors |  |

465. What prevented the control from being identified? Select all that apply.  [List Checkbox]

| ☐ Lack of familiarity with specific high energy hazards - The project team is not familiar with the specific high energy hazards present in the task, i.e. new or unique situation or work process. | ☐ Inadequate training on high energy controls - Team members have not received sufficient training on identifying, implementing, and executing high energy controls. |
| --- | --- |
| ☐ Insufficient risk assessment - The team has not conducted a thorough risk assessment to identify all potential high energy hazards and determine appropriate controls. | ☐ Lack of available control options - The team struggles to find suitable control measures for mitigating the identified high energy hazards, leading to delays or inadequate solutions. |
| ☐ Resource constraints - Limited budget or access to equipment prevented the implementation of high energy controls | ☐ Time constraints - Project deadlines and tight schedules pressure the team to overlook or rush the identification of high energy controls. |
| ☐ Risk Tolerance - The team has dealt with similar high energy hazards in the past without incident and assumed the risks are manageable without additional controls. | ☐ Emphasis on production - A focus on productivity and meeting targets led to neglecting the thorough identification of high energy controls. |
| ☐ Lack of communication and collaboration - Team members did not communicate effectively, critical information about high energy hazards and controls were missed. | ☐ Failure to learn from past incidents - The team did not learn from previous incidents involving similar high energy hazards, leading to repeated oversights in control identification. |
| ☐ Complexity of the construction task - Highly complex task with multiple high energy hazards, made it challenging for the team to identify and implement appropriate controls for each hazard. |  |

466. What prevented the control from being implemented? Select all that apply.  [List Checkbox]

| ☐ Cost considerations - High energy controls are expensive to acquire and install, and the project team prioritized cost savings over worker safety. | ☐ Lack of understanding or awareness - The team did not fully comprehend the severity of the hazard or the potential consequences of not implementing the high energy controls. |
| --- | --- |
| ☐ Resistance to change - Introduction of controls or procedures was met with resistance from workers or management who prefer familiar practices. | ☐ Inadequate planning - The team identified the need for high energy control but failed to develop a proper plan for their implementation. |
| ☐ Resource constraints - The team lacked the necessary resources, such as equipment or skilled personnel, to implement the high energy controls effectively. | ☐ Time constraints - Project timelines or deadlines did not allow for implementing high energy controls properly. |
| ☐ Lack of leadership support - The absence of leadership support in prioritizing control implementation made it challenging for the team to effectively implement the controls. | ☐ Perceived inconvenience - Workers considered the high energy controls as inconvenient or cumbersome, leading to resistance in their implementation. |
| ☐ Risk Tolerance - The project team believes that the hazard is manageable without additional controls or that accidents are unlikely to happen. | ☐ Inadequate training - Workers lack the necessary training on how to use or maintain the high energy controls effectively. |
| ☐ Miscommunication or misinterpretation - Instructions or guidelines for implementing the controls have not been communicated clearly or were misunderstood by the team |  |

467. What prevented the control from being executed? Select all that apply.  [List Checkbox]

| ☐ Inadequate training - The team have not received sufficient training on how to use the high energy control properly | ☐ Lack of supervision - Lacking supervision, workers did not follow the correct procedures for using the high energy control. |
| --- | --- |
| ☐ Risk Tolerance - Workers became complacent over time and deviated from established procedures | ☐ Lack of accountability - No consequences for non-compliance with high energy control procedures |
| ☐ Failure to maintain equipment - Lack of regular maintenance and/or inspections of the high energy control led to malfunction and/or reduced effectiveness. | ☐ Miscommunication or misunderstandings - Poor communication between team members resulted in misunderstandings about how to execute the high energy control. |
| ☐ Production pressures - Workers prioritized production over control execution, due to tight project schedule or deadlines, leading to incomplete execution of the high energy control. | ☐ Resource constraints - The team lacked the necessary resources to ensure high energy control execution. |
| ☐ Lack of awareness - Workers did understand the importance of high energy control or the potential consequences of not executing it effectively. | ☐ Worker fatigue - Fatigued workers were not alert or attentive when executing high energy controls. |
| ☐ Competing priorities - Other tasks or issues diverted the team’s attention away from executing the high energy control. | ☐ Willful disregard - Intentional neglect or avoidance of necessary actions or procedures |

468. Adequate execution of lift plan? (Cranes)  [List Radio]

Known weight of load
Known Capacity of load handling equipment
Anti-Two-Block Functional
Rigging inspected and adequate for task
Controlled lift zone
Tag lines
Designated signal person
Qualified rigger
Overhead load alert

○ Yes

○ No

469. Execution of lift plan - Corrective Actions  [List Checkbox]

| ☐ Determine/Verify weight of load | ☐ Determine capacity of load handling equipment |
| --- | --- |
| ☐ Install Anti-Two-Block device | ☐ Replace rigging |
| ☐ Establish controlled lift zone | ☐ Provide tag lines |
| ☐ Designate qualified signal person | ☐ Assign qualified rigger |
| ☐ Establish overhead load alert system | ☐ Use the correct signal method |

470. Reason for lack of executing lift plan (Cranes)  [List Radio]

○ Load Handling hazard(s) not recognized

○ Load Handling control(s) not identified

○ Load Handling control(s) not implemented

○ Load Handling control(s) not executed

471. What prevented the hazard from being recognized? Select all that apply  [List Checkbox]

| ☐ Primary STCKY Energy not recognized | ☐ Lack of Knowledge |
| --- | --- |
| ☐ Inadequate training | ☐ Lack of awareness |
| ☐ Insufficient risk assessment | ☐ Rushed planning or inadequate preparation |
| ☐ Lack of communication | ☐ Inadequate hazard identification process |
| ☐ Unclear responsibilities | ☐ Assumptions and biases |
| ☐ External factors |  |

472. What prevented the control from being identified? Select all that apply.  [List Checkbox]

| ☐ Lack of familiarity with specific high energy hazards - The project team is not familiar with the specific high energy hazards present in the task, i.e. new or unique situation or work process. | ☐ Inadequate training on high energy controls - Team members have not received sufficient training on identifying, implementing, and executing high energy controls. |
| --- | --- |
| ☐ Insufficient risk assessment - The team has not conducted a thorough risk assessment to identify all potential high energy hazards and determine appropriate controls. | ☐ Lack of available control options - The team struggles to find suitable control measures for mitigating the identified high energy hazards, leading to delays or inadequate solutions. |
| ☐ Resource constraints - Limited budget or access to equipment prevented the implementation of high energy controls | ☐ Time constraints - Project deadlines and tight schedules pressure the team to overlook or rush the identification of high energy controls. |
| ☐ Risk Tolerance - The team has dealt with similar high energy hazards in the past without incident and assumed the risks are manageable without additional controls. | ☐ Emphasis on production - A focus on productivity and meeting targets led to neglecting the thorough identification of high energy controls. |
| ☐ Lack of communication and collaboration - Team members did not communicate effectively, critical information about high energy hazards and controls were missed. | ☐ Failure to learn from past incidents - The team did not learn from previous incidents involving similar high energy hazards, leading to repeated oversights in control identification. |
| ☐ Complexity of the construction task - Highly complex task with multiple high energy hazards, made it challenging for the team to identify and implement appropriate controls for each hazard. |  |

473. What prevented the control from being implemented? Select all that apply.  [List Checkbox]

| ☐ Cost considerations - High energy controls are expensive to acquire and install, and the project team prioritized cost savings over worker safety. | ☐ Lack of understanding or awareness - The team did not fully comprehend the severity of the hazard or the potential consequences of not implementing the high energy controls. |
| --- | --- |
| ☐ Resistance to change - Introduction of controls or procedures was met with resistance from workers or management who prefer familiar practices. | ☐ Inadequate planning - The team identified the need for high energy control but failed to develop a proper plan for their implementation. |
| ☐ Resource constraints - The team lacked the necessary resources, such as equipment or skilled personnel, to implement the high energy controls effectively. | ☐ Time constraints - Project timelines or deadlines did not allow for implementing high energy controls properly. |
| ☐ Lack of leadership support - The absence of leadership support in prioritizing control implementation made it challenging for the team to effectively implement the controls. | ☐ Perceived inconvenience - Workers considered the high energy controls as inconvenient or cumbersome, leading to resistance in their implementation. |
| ☐ Risk Tolerance - The project team believes that the hazard is manageable without additional controls or that accidents are unlikely to happen. | ☐ Inadequate training - Workers lack the necessary training on how to use or maintain the high energy controls effectively. |
| ☐ Miscommunication or misinterpretation - Instructions or guidelines for implementing the controls have not been communicated clearly or were misunderstood by the team |  |

474. What prevented the control from being executed? Select all that apply.  [List Checkbox]

| ☐ Inadequate training - The team have not received sufficient training on how to use the high energy control properly | ☐ Lack of supervision - Lacking supervision, workers did not follow the correct procedures for using the high energy control. |
| --- | --- |
| ☐ Risk Tolerance - Workers became complacent over time and deviated from established procedures | ☐ Lack of accountability - No consequences for non-compliance with high energy control procedures |
| ☐ Failure to maintain equipment - Lack of regular maintenance and/or inspections of the high energy control led to malfunction and/or reduced effectiveness. | ☐ Miscommunication or misunderstandings - Poor communication between team members resulted in misunderstandings about how to execute the high energy control. |
| ☐ Production pressures - Workers prioritized production over control execution, due to tight project schedule or deadlines, leading to incomplete execution of the high energy control. | ☐ Resource constraints - The team lacked the necessary resources to ensure high energy control execution. |
| ☐ Lack of awareness - Workers did understand the importance of high energy control or the potential consequences of not executing it effectively. | ☐ Worker fatigue - Fatigued workers were not alert or attentive when executing high energy controls. |
| ☐ Competing priorities - Other tasks or issues diverted the team’s attention away from executing the high energy control. | ☐ Willful disregard - Intentional neglect or avoidance of necessary actions or procedures |

475. Required Load Handling Equipment set up procedures in place? (Other than Cranes)?  [List Radio]

Ground conditions sufficient for task
Outriggers extended as required by load chart
Lifting equipment level during lift
Spotter or C.A.Z in place as needed
Using manufacturer approved lifting points

○ Yes

○ No

476. Load Handling Equipment set up procedures - Corrective Actions  [List Checkbox]

☐ Improve ground conditions

☐ Extend outriggers per load chart

☐ Ensure lifting equipment is level

☐ Set up Spotter or C.A.Z as needed

☐ Use manufacturer approved lifting points

477. Reason for lack of set up procedures for load handling equipment  [List Radio]

○ Lift Set Up hazard(s) not recognized

○ Lift Set Up control(s) not identified

○ Lift Set Up control(s) not implemented

○ Lift Set Up control(s) not executed

478. What prevented the hazard from being recognized? Select all that apply.  [List Checkbox]

| ☐ Primary STCKY Energy not recognized | ☐ Lack of Knowledge |
| --- | --- |
| ☐ Inadequate training | ☐ Lack of awareness |
| ☐ Insufficient risk assessment | ☐ Rushed planning or inadequate preparation |
| ☐ Lack of communication | ☐ Inadequate hazard identification process |
| ☐ Unclear responsibilities | ☐ Assumptions and biases |
| ☐ External factors |  |

479. What prevented the control from being identified? Select all that apply.  [List Checkbox]

| ☐ Lack of familiarity with specific high energy hazards - The project team is not familiar with the specific high energy hazards present in the task, i.e. new or unique situation or work process. | ☐ Inadequate training on high energy controls - Team members have not received sufficient training on identifying, implementing, and executing high energy controls. |
| --- | --- |
| ☐ Insufficient risk assessment - The team has not conducted a thorough risk assessment to identify all potential high energy hazards and determine appropriate controls. | ☐ Lack of available control options - The team struggles to find suitable control measures for mitigating the identified high energy hazards, leading to delays or inadequate solutions. |
| ☐ Resource constraints - Limited budget or access to equipment prevented the implementation of high energy controls | ☐ Time constraints - Project deadlines and tight schedules pressure the team to overlook or rush the identification of high energy controls. |
| ☐ Risk Tolerance - The team has dealt with similar high energy hazards in the past without incident and assumed the risks are manageable without additional controls. | ☐ Emphasis on production - A focus on productivity and meeting targets led to neglecting the thorough identification of high energy controls. |
| ☐ Lack of communication and collaboration - Team members did not communicate effectively, critical information about high energy hazards and controls were missed. | ☐ Failure to learn from past incidents - The team did not learn from previous incidents involving similar high energy hazards, leading to repeated oversights in control identification. |
| ☐ Complexity of the construction task - Highly complex task with multiple high energy hazards, made it challenging for the team to identify and implement appropriate controls for each hazard. |  |

480. What prevented the control from being implemented? Select all that apply.  [List Checkbox]

| ☐ Cost considerations - High energy controls are expensive to acquire and install, and the project team prioritized cost savings over worker safety. | ☐ Lack of understanding or awareness - The team did not fully comprehend the severity of the hazard or the potential consequences of not implementing the high energy controls. |
| --- | --- |
| ☐ Resistance to change - Introduction of controls or procedures was met with resistance from workers or management who prefer familiar practices. | ☐ Inadequate planning - The team identified the need for high energy control but failed to develop a proper plan for their implementation. |
| ☐ Resource constraints - The team lacked the necessary resources, such as equipment or skilled personnel, to implement the high energy controls effectively. | ☐ Time constraints - Project timelines or deadlines did not allow for implementing high energy controls properly. |
| ☐ Lack of leadership support - The absence of leadership support in prioritizing control implementation made it challenging for the team to effectively implement the controls. | ☐ Perceived inconvenience - Workers considered the high energy controls as inconvenient or cumbersome, leading to resistance in their implementation. |
| ☐ Risk Tolerance - The project team believes that the hazard is manageable without additional controls or that accidents are unlikely to happen. | ☐ Inadequate training - Workers lack the necessary training on how to use or maintain the high energy controls effectively. |
| ☐ Miscommunication or misinterpretation - Instructions or guidelines for implementing the controls have not been communicated clearly or were misunderstood by the team |  |

481. What prevented the control from being executed? Select all that apply.  [List Checkbox]

| ☐ Inadequate training - The team have not received sufficient training on how to use the high energy control properly | ☐ Lack of supervision - Lacking supervision, workers did not follow the correct procedures for using the high energy control. |
| --- | --- |
| ☐ Risk Tolerance - Workers became complacent over time and deviated from established procedures | ☐ Lack of accountability - No consequences for non-compliance with high energy control procedures |
| ☐ Failure to maintain equipment - Lack of regular maintenance and/or inspections of the high energy control led to malfunction and/or reduced effectiveness. | ☐ Miscommunication or misunderstandings - Poor communication between team members resulted in misunderstandings about how to execute the high energy control. |
| ☐ Production pressures - Workers prioritized production over control execution, due to tight project schedule or deadlines, leading to incomplete execution of the high energy control. | ☐ Resource constraints - The team lacked the necessary resources to ensure high energy control execution. |
| ☐ Lack of awareness - Workers did understand the importance of high energy control or the potential consequences of not executing it effectively. | ☐ Worker fatigue - Fatigued workers were not alert or attentive when executing high energy controls. |
| ☐ Competing priorities - Other tasks or issues diverted the team’s attention away from executing the high energy control. | ☐ Willful disregard - Intentional neglect or avoidance of necessary actions or procedures |

482. Safe procedures for work near overhead utilities (OLHD)?  [List Radio]

Overhead Utilities (OLHD):
Encroachment Precautions for operations up to 350kV:
All of the following must be met:
Planning meeting (discuss de-energizing lines)
Signs and barricades
Non-conductive tag lines
Warning lines erected
Spotter required
Table A – Minimum Powerline Clearance Distances – Operation
Voltage
Distance
0 to 50 kV
10 ft
50 to 200 kV
15 ft
200 to 350 kV
20 ft
350 to 500 kV
25 ft
500 to 750 kV
35 ft
750 to 1000 kV
45 ft

○ Yes

○ No

○ Not applicable

483. Work near overhead utilities (OLHD) - Corrective Actions  [List Checkbox]

☐ Conduct planning meeting (discuss de-energizing lines)

☐ Erect signs and barricades

☐ Erect warning lines

☐ Provide Non-conductive tag lines

☐ Designate spotter as required

☐ Ensure minimum clearance distance per Table A

484. Reason for lack of procedures while lifting near overhead utilities (OLHD)  [List Radio]

○ Overhead Utilities hazard(s) not recognized

○ Overhead Utilities control(s) not identified

○ Overhead Utilities control(s) not implemented

○ Overhead Utilities control(s) not executed

485. What prevented the hazard from being recognized? Select all that apply.  [List Checkbox]

| ☐ Primary STCKY Energy not recognized | ☐ Lack of Knowledge |
| --- | --- |
| ☐ Inadequate training | ☐ Lack of awareness |
| ☐ Insufficient risk assessment | ☐ Rushed planning or inadequate preparation |
| ☐ Lack of communication | ☐ Inadequate hazard identification process |
| ☐ Unclear responsibilities | ☐ Assumptions and biases |
| ☐ External factors |  |

486. What prevented the control from being identified? Select all that apply.  [List Checkbox]

| ☐ Lack of familiarity with specific high energy hazards - The project team is not familiar with the specific high energy hazards present in the task, i.e. new or unique situation or work process. | ☐ Inadequate training on high energy controls - Team members have not received sufficient training on identifying, implementing, and executing high energy controls. |
| --- | --- |
| ☐ Insufficient risk assessment - The team has not conducted a thorough risk assessment to identify all potential high energy hazards and determine appropriate controls. | ☐ Lack of available control options - The team struggles to find suitable control measures for mitigating the identified high energy hazards, leading to delays or inadequate solutions. |
| ☐ Resource constraints - Limited budget or access to equipment prevented the implementation of high energy controls | ☐ Time constraints - Project deadlines and tight schedules pressure the team to overlook or rush the identification of high energy controls. |
| ☐ Risk Tolerance - The team has dealt with similar high energy hazards in the past without incident and assumed the risks are manageable without additional controls. | ☐ Emphasis on production - A focus on productivity and meeting targets led to neglecting the thorough identification of high energy controls. |
| ☐ Lack of communication and collaboration - Team members did not communicate effectively, critical information about high energy hazards and controls were missed. | ☐ Failure to learn from past incidents - The team did not learn from previous incidents involving similar high energy hazards, leading to repeated oversights in control identification. |
| ☐ Complexity of the construction task - Highly complex task with multiple high energy hazards, made it challenging for the team to identify and implement appropriate controls for each hazard. |  |

487. What prevented the control from being implemented? Select all that apply.  [List Checkbox]

| ☐ Cost considerations - High energy controls are expensive to acquire and install, and the project team prioritized cost savings over worker safety. | ☐ Lack of understanding or awareness - The team did not fully comprehend the severity of the hazard or the potential consequences of not implementing the high energy controls. |
| --- | --- |
| ☐ Resistance to change - Introduction of controls or procedures was met with resistance from workers or management who prefer familiar practices. | ☐ Inadequate planning - The team identified the need for high energy control but failed to develop a proper plan for their implementation. |
| ☐ Resource constraints - The team lacked the necessary resources, such as equipment or skilled personnel, to implement the high energy controls effectively. | ☐ Time constraints - Project timelines or deadlines did not allow for implementing high energy controls properly. |
| ☐ Lack of leadership support - The absence of leadership support in prioritizing control implementation made it challenging for the team to effectively implement the controls. | ☐ Perceived inconvenience - Workers considered the high energy controls as inconvenient or cumbersome, leading to resistance in their implementation. |
| ☐ Risk Tolerance - The project team believes that the hazard is manageable without additional controls or that accidents are unlikely to happen. | ☐ Inadequate training - Workers lack the necessary training on how to use or maintain the high energy controls effectively. |
| ☐ Miscommunication or misinterpretation - Instructions or guidelines for implementing the controls have not been communicated clearly or were misunderstood by the team |  |

488. What prevented the control from being executed? Select all that apply.  [List Checkbox]

| ☐ Inadequate training - The team have not received sufficient training on how to use the high energy control properly | ☐ Lack of supervision - Lacking supervision, workers did not follow the correct procedures for using the high energy control. |
| --- | --- |
| ☐ Risk Tolerance - Workers became complacent over time and deviated from established procedures | ☐ Lack of accountability - No consequences for non-compliance with high energy control procedures |
| ☐ Failure to maintain equipment - Lack of regular maintenance and/or inspections of the high energy control led to malfunction and/or reduced effectiveness. | ☐ Miscommunication or misunderstandings - Poor communication between team members resulted in misunderstandings about how to execute the high energy control. |
| ☐ Production pressures - Workers prioritized production over control execution, due to tight project schedule or deadlines, leading to incomplete execution of the high energy control. | ☐ Resource constraints - The team lacked the necessary resources to ensure high energy control execution. |
| ☐ Lack of awareness - Workers did understand the importance of high energy control or the potential consequences of not executing it effectively. | ☐ Worker fatigue - Fatigued workers were not alert or attentive when executing high energy controls. |
| ☐ Competing priorities - Other tasks or issues diverted the team’s attention away from executing the high energy control. | ☐ Willful disregard - Intentional neglect or avoidance of necessary actions or procedures |

489. Adequate execution of lift procedures? (OLHD)  [List Radio]

Known weight of load
Known Capacity of load handling equipment
Rigging inspected and adequate for task
Controlled lift zone
Tag lines as required
Designated signal person as required
Qualified rigger as required

○ Yes

○ No

○ Not applicable

490. Execution of lift procedures - Corrective Actions  [List Checkbox]

☐ Determine/Verify weight of load

☐ Determine capacity of load handling equipment

☐ Replace rigging

☐ Establish controlled lift zone

☐ Provide tag lines as required

☐ Designate qualified signal person as required

☐ Assign qualified rigger as required

491. Reason for lack of safe lifting  [List Radio]

○ Load Handling hazard(s) not recognized

○ Load Handling control(s) not identified

○ Load Handling control(s) not implemented

○ Load Handling control(s) not executed

492. What prevented the hazard from being recognized? Select all that apply.  [List Checkbox]

| ☐ Primary STCKY Energy not recognized | ☐ Lack of Knowledge |
| --- | --- |
| ☐ Inadequate training | ☐ Lack of awareness |
| ☐ Insufficient risk assessment | ☐ Rushed planning or inadequate preparation |
| ☐ Lack of communication | ☐ Inadequate hazard identification process |
| ☐ Unclear responsibilities | ☐ Assumptions and biases |
| ☐ External factors |  |

493. What prevented the control from being identified? Select all that apply.  [List Checkbox]

| ☐ Lack of familiarity with specific high energy hazards - The project team is not familiar with the specific high energy hazards present in the task, i.e. new or unique situation or work process. | ☐ Inadequate training on high energy controls - Team members have not received sufficient training on identifying, implementing, and executing high energy controls. |
| --- | --- |
| ☐ Insufficient risk assessment - The team has not conducted a thorough risk assessment to identify all potential high energy hazards and determine appropriate controls. | ☐ Lack of available control options - The team struggles to find suitable control measures for mitigating the identified high energy hazards, leading to delays or inadequate solutions. |
| ☐ Resource constraints - Limited budget or access to equipment prevented the implementation of high energy controls | ☐ Time constraints - Project deadlines and tight schedules pressure the team to overlook or rush the identification of high energy controls. |
| ☐ Risk Tolerance - The team has dealt with similar high energy hazards in the past without incident and assumed the risks are manageable without additional controls. | ☐ Emphasis on production - A focus on productivity and meeting targets led to neglecting the thorough identification of high energy controls. |
| ☐ Lack of communication and collaboration - Team members did not communicate effectively, critical information about high energy hazards and controls were missed. | ☐ Failure to learn from past incidents - The team did not learn from previous incidents involving similar high energy hazards, leading to repeated oversights in control identification. |
| ☐ Complexity of the construction task - Highly complex task with multiple high energy hazards, made it challenging for the team to identify and implement appropriate controls for each hazard. |  |

494. What prevented the control from being implemented? Select all that apply.  [List Checkbox]

| ☐ Cost considerations - High energy controls are expensive to acquire and install, and the project team prioritized cost savings over worker safety. | ☐ Lack of understanding or awareness - The team did not fully comprehend the severity of the hazard or the potential consequences of not implementing the high energy controls. |
| --- | --- |
| ☐ Resistance to change - Introduction of controls or procedures was met with resistance from workers or management who prefer familiar practices. | ☐ Inadequate planning - The team identified the need for high energy control but failed to develop a proper plan for their implementation. |
| ☐ Resource constraints - The team lacked the necessary resources, such as equipment or skilled personnel, to implement the high energy controls effectively. | ☐ Time constraints - Project timelines or deadlines did not allow for implementing high energy controls properly. |
| ☐ Lack of leadership support - The absence of leadership support in prioritizing control implementation made it challenging for the team to effectively implement the controls. | ☐ Perceived inconvenience - Workers considered the high energy controls as inconvenient or cumbersome, leading to resistance in their implementation. |
| ☐ Risk Tolerance - The project team believes that the hazard is manageable without additional controls or that accidents are unlikely to happen. | ☐ Inadequate training - Workers lack the necessary training on how to use or maintain the high energy controls effectively. |
| ☐ Miscommunication or misinterpretation - Instructions or guidelines for implementing the controls have not been communicated clearly or were misunderstood by the team |  |

495. What prevented the control from being executed? Select all that apply.  [List Checkbox]

| ☐ Inadequate training - The team have not received sufficient training on how to use the high energy control properly | ☐ Lack of supervision - Lacking supervision, workers did not follow the correct procedures for using the high energy control. |
| --- | --- |
| ☐ Risk Tolerance - Workers became complacent over time and deviated from established procedures | ☐ Lack of accountability - No consequences for non-compliance with high energy control procedures |
| ☐ Failure to maintain equipment - Lack of regular maintenance and/or inspections of the high energy control led to malfunction and/or reduced effectiveness. | ☐ Miscommunication or misunderstandings - Poor communication between team members resulted in misunderstandings about how to execute the high energy control. |
| ☐ Production pressures - Workers prioritized production over control execution, due to tight project schedule or deadlines, leading to incomplete execution of the high energy control. | ☐ Resource constraints - The team lacked the necessary resources to ensure high energy control execution. |
| ☐ Lack of awareness - Workers did understand the importance of high energy control or the potential consequences of not executing it effectively. | ☐ Worker fatigue - Fatigued workers were not alert or attentive when executing high energy controls. |
| ☐ Competing priorities - Other tasks or issues diverted the team’s attention away from executing the high energy control. | ☐ Willful disregard - Intentional neglect or avoidance of necessary actions or procedures |