| Each supervisor will review the Work Package and this JHA with the personnel assigned to perform or otherwise engage in this scope of work. This JHA shall be maintained in the field at the site of activity and be revised in real time if scope, conditions, or hazards change. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Scope of Work: |  |  |  |  |
| Required Permits/Plans: ☐ General Work Permit ☐ Confined Space Entry Permit ☐ Crane Pick Plans/Crit. Permit ☐ Daily Excavation Log ☐ Demolition Checklist ☐ Disturbance Permit ☐ Energized Electr. Work Permit ☐ Fall Protection Plan/Stop the Drop/Elevated Work Permit ☐ HEC/LOTO Permit ☐ Hot Work Permit ☐ Line Break Permit ☐ Pneumatic Test Permit Other________________________ | Specialty Tools & Mobile Equipment Required: - __________________________ - __________________________ - __________________________ - __________________________ - __________________________ - __________________________ - __________________________ - __________________________ - __________________________ - _________________________ | Additional PPE required: ☐ Hand (Glove Type) __________ ☐ Arm _______________________ ☐ Leg ________________________ ☐ Foot ☐ Hearing ____________________ ☐ Face (Goggle or Shield) _______ ☐ Fall Protection ______________ ☐ Respirator __________________ ☐ Tool/material tethers _________ Other _____________________ Other _____________________ | Environmental: ☐ Extreme Heat ☐ Extreme Cold ☐ Air Quality Concerns ☐ Flooding ☐ Lightning ☐ High Winds ☐ Spiders/Insects ☐ Snakes/Wildlife ☐ Chemicals ☐ Major Spill Potential ☐ Operating plant equip in sensitive area ☐ Other | Identify potential STCKY Energy: |

| Actions & Steps | Hazards | Controls to Implement & Execute | Risk Rating | CONTROL VERIFICATION |
| --- | --- | --- | --- | --- |
| Responding to an incident or emergency | - Becoming an additional casualty | - All Injuries shall be reported to Sundt Management - In the event a serious incident, all personnel will assemble to a pre-designated area, which will be identified by the foremen during the THA review. - The Sundt superintendent will take charge of the scene - If media is on site personnel shall not talk to media - Air horn is the signal to evacuate and meet at the designated meeting point - Secure equipment and loads prior to evacuating. | A |  |
| Working in hot conditions | - Heat Illness | - Provide workers with adequate water - Discuss the importance of water consumption – 1 quart per hour. - Foremen can replenish water supply and if needed water can be found in the trailer. - Workers are encouraged to seek shade during all breaks and lunch. - If needed workers can access trailer for shade. - Look for signs of dehydration during the work day. If you see issues notify supervision - Emergency situation, loosen clothing, move to cool area, give water and use cool packs underarms and in groin. - Know the symptoms - Heat Cramps – Muscle spasms - Heat exhaustion – Dizziness, headache, sweaty skin, weakness, cramps, vomiting, fast heartbeat. - Heat Stroke – red hot dry skin, confusion, convulsions, fainting - Take immediate action and report any of these symptoms to supervision. | B |  |
| Handling material by hand | - Muscle strain/sprain - Laceration - Crush - Caught in - Struck by - Abrasion | - Stretch and flex at start of shift - When lifting use your legs, have clear path of travel, - Avoid stepping over or up/down when carrying items - Use two people for lifts over 50 pounds or use mechanical methods to move items - When possible, don’t work over head or stooped down. - Know path of travel and location of where material is to be staged. - Bring material and work at waist level. - 100% glove use required at all times. - Cut-resistant gloves and sleeves required for handling sharp material - Rule of Thumb: If you can’t see your fingers then they are risk of potential pinch point or crush - Use extensions like 2x4 of hammer to avoid placing hand ins potential pinch points - Use handle and guards with all tools. - Always use sharp knives and cut away from yourself. - Never use a tool that it was not intended for. - Choose gloves that are appropriate for the task. |  |  |
| Working with chemicals | - Illness / exposure | - Review SDSs for PPE requirements and exposure limitations - Provide ventilation when needed - Conduct atmospheric testing or have a 3rd party conduct testing if needed - Use wet methods to control dust - Hearing protection required when noises are too loud. |  |  |
| Using ladders | - Falls - Overreaching - Tip over from improper set up - Improper use per design of ladder - Damages - Twisting bending / body position | - Always use ladder per manufactures design - Visually inspect ladder before each use and quarterly with GORY - Set on level surface - Keep body within frame of ladder - Use podium ladder as needed |  |  |
| Working from Elevating Work Platforms | - Falls - Equipment failure - Chemical spills - Pinch points - Basket swaying excessively - Uneven surface - Others overhead or below - Rebar impalement - Striking other equipment/ structures - Openings | - Climbing on rails or toe board is prohibited by SUNDT - Reposition the basket to have proper body position for the work you are performing - Manufacturer approval is required in order to exit the work platform from an elevated position - 100% tie off to proper anchor point (boom lifts) - Visually inspect and document inspection daily - Initiate Emergency response for spills/spill kit in office - Mind hand placement during movement of lift - Operate/travel as low to the ground as possible - Survey the area you are about to enter - Communicate with other trades in the area - Verify rebar protection is in place in the area immediately below you - Identify equipment and structures and maintain safe distance typically 10’, when feasible - Identify trenches and keep |  |  |
| Disposal of waste material | - Pinch point for hands - Heavy equipment: forklift, skid steer - Tip over of 40-yard dumpster | - Workers must be cautious of pinch points during waste handling—pay attention to the release on the metal hoppers - Only trained and certified operators are allowed to operate the skid or forklift - Seatbelts/restraints to be worn during operation - Daily inspections shall be conducted prior to operating - Equipment routes to be established and protected to prevent personnel from entering the area as feasible - 40 yard dumpsters to be located in a manner that allows for even loading and safe removal from the project |  |  |
| Housekeeping | - Slip/trip/fall - Laceration/ abrasion to hands/arms - Muscle strain/sprain - Fire - Chemical spill | At all times: - Floors shall be maintained in a broom-swept condition. - Egress pathways shall be free of materials and/or debris. - Material located indoors shall be kept on rolling carts or pallets. - Objects on wheels that have the potential to fall to lower levels must have brakes and/or tethers. - Waste shall be placed into trash receptacles immediately (no trash shall be left on the ground). - No food or beverages other than water are permitted outside of Sundt-designated break areas. - Pallets shall be marked with the contractor’s company name (or follow site-specific requirements for labeling/color coding). - Coordination between Sundt personnel and foreman/superintendent must occur for all movement of materials from lay down to working areas. Stored materials must: - Not clutter work areas. - Not block other trades’ access to their work. At the end of each day: - Cords and hoses shall be rolled up and stored. - Tools shall be properly stored in gang boxes. - Work areas shall be broom swept using controls to prevent the creation of airborne dust. |  |  |
| Working Overhead | - Dropped objects | Tethering/Securing of Drop Hazards - Tools and materials must be tethered/secured when screening or other controls will not prevent a drop hazard. This requirement covers all work areas and is not limited to structure perimeters or leading edges. Therefore, this requirement applies to wherever potential risk to others exists from dropped/falling objects. - Prior to selecting a tool lanyard, a proper attachment point must be established on the tool and the person/belt/harness/equipment. Attachment points must be manufactured for the purpose of tethering or certified as third-party tested for dynamic load by the manufacturer. Attachment points must be tested to an adequate load rating for the intended use of the tool. - Tethers/lanyards shall be inspected prior to use. Excessively worn or damaged tools or materials must be immediately removed from service and replaced. - Tools weighing more than five pounds may not be tethered directly to a person’s body or wristband. - Wristbands used for tethering must be certified by the manufacturer as having been third-party tested for dynamic load. - Positive tool transfer must be utilized. (When transferring a tethered tool from one person to another, “100% tie off” must be engaged. The tool must be tethered to the passing person. Prior to handing off, the receiving person must connect their tether to the tool as well. After positive connection has been completed, the passing person may disconnect their tether from the tool.) - For the safe transportation of tools and materials, buckets/holsters/pouches/bags may be utilized only if they are manufactured with a closure system which allows the user to secure the contents from potential spills. Buckets/bags that are hoisted must be load rated by the manufacturer and third-party tested for static load. Controlled Access Zones (CAZs) - CAZs and/or designated spotters must be in place unless measures are enacted to prevent drop hazards (i.e., screening, netting, etc.). - CAZs must be clearly marked with barricades or danger tape to restrict access to unauthorized personnel. When a CAZ is no longer in effect, the barricades/tape must be taken down. - CAZs must have signage posted that identifies who the CAZ belongs to, what the hazard is, and the person to contact for permission to access the area. - CAZs must be of adequate size to effectively mitigate the risk from falling or ricocheting material. - Persons authorized to work in CAZs must pay attention to what is going on above them (i.e., scaffold erection/dismantling, concrete deck forming, installation/removal of perimeter protection, etc.). - Persons not authorized to enter a CAZ may be subject to disciplinary procedures which may include termination. - CAZs must be in place around hoisting activities (i.e., structural steel shake-out areas, forklift loading zones, etc.). Paths of travel for overhead loads must be kept clear of persons. Storage of Drop Hazards - Trash and waste must be properly contained in buckets/pouches/containers that have the ability to be closed to prevent spillage. - Work areas, tools, and materials must be maintained in an orderly fashion. - Unless guardrails with screening or paneling have been erected, unsecured tools/materials must not be stored within 15 feet of a leading/perimeter edge. - Stacked materials must be stable and self-supporting. - Potential drop hazards must be secured to prevent them from being wind-blown. |  |  |
| Walking/ working below overhead activities | - Struck by - Crush - Caught in/ between | - Stay out of CAZs unless authorized to access and work overhead has ceased - Coordinate and communicate with other workers to prevent vertical trade stacking - Be aware of and listen for workers overhead - Do not walk under overhead activities and give them adequate room |  |  |

MANAGEMENT REVIEW

|  | Print Name | Signature | Date |
| --- | --- | --- | --- |
| Superintendent: |  |  |  |
| HSE Manager: |  |  |  |
| General Superintendent: |  |  |  |
| Project Manager: |  |  |  |

CREW REVIEW

| Print Name | Foreman | Date | Print Name | Foreman | Date |
| --- | --- | --- | --- | --- | --- |

AUDIT

| Item | Adequate | Inadequate | Comments |
| --- | --- | --- | --- |
| Scope of work description | ☐ | ☐ |  |
| Permits, Tools, PPE, Environmental | ☐ | ☐ |  |
| Scope STCKY | ☐ | ☐ |  |
| Tasks | ☐ | ☐ |  |
| Hazards | ☐ | ☐ |  |
| Hazard controls | ☐ | ☐ |  |
| Close proximity | ☐ | ☐ |  |
| Task/Potential STCKY | ☐ | ☐ |  |
| Controls field verified | ☐ | ☐ |  |
| Management review | ☐ | ☐ |  |
| Crew review | ☐ | ☐ |  |
| JHA in work location | ☐ | ☐ |  |
| Additional Comments: |  |  |  |

Auditor Information

| Print Name |  | Signature |  | Date |
| --- | --- | --- | --- | --- |
| Print Name |  | Signature |  | Date |
| Print Name |  | Signature |  | Date |

RISK RATING KEY

Use the following information and key to assess the risk for each task. Risk shall be assessed based on available control methods in place.

Frequency of Task

| 1 | Infrequent | Possible the activity will occur, but not likely. |
| --- | --- | --- |
| 2 | Occasional | Likelihood of activity occurring 1-3 times during scope of work. |
| 3 | Frequent | Possibility that activity will 4 or more times during scope of work, but not daily. |
| 4 | Very Frequent | Possibility of repeated activity (at least daily). |

Severity of Consequences

| Category |  | People | Property | Environment | Public Image, Reputation, Disruption |
| --- | --- | --- | --- | --- | --- |
| 1 | Minor | First Aid/No injury | Impact <$10K | No impact | Individual or none |
| 2 | Serious | Recordable Injury | Impact >$10K but <$50K | Spill less than allowable by permit/law | Community attention |
| 3 | Critical | Permanent or long-term injury or illness | Impact >$50K but <$100K | Client standards not met | Owner intervention |
| 4 | Catastrophic | Fatality | Impact >$100K | Reportable event (NOV) | Government intervention |

|  |  | Frequency of Task |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 |  | Risk Rating |  | Action Required |
| Severity | 1 | 1 | 2 | 3 | 4 |  | A | Low (1-3) | Can be managed at field level with appropriate mitigation in place. |
|  | 2 | 2 | 4 | 6 | 8 |  | B | Medium (4-6) | Approval required by 2 members of project management. |
|  | 3 | 3 | 6 | 9 | 12 |  | C | High (8-16) | Cannot continue with activity; must have approval of Group HSE Manager and Project Director or Sector Manager. |
|  | 4 | 4 | 8 | 12 | 16 |  |  |  |  |